| To: | eDriveXD / eDriveXC Users | Date: | March 21, 2018 |
| --- | --- | --- | --- |
| From: | Global Customer Care | Subject: | eDriveXD / eDriveXC v1.7Recovery Update Instructions |

Product(s) Affected:

eDriveXD / eDriveXC

Scope:

This document provides software installation instructions for Outback eDriveXD / eDriveXC v1.7 Recovery. This software allows the user to update the steering ECU’s software without be connected to an Outback terminal.

To take advantage of the improvements listed above, visit www.OutbackGuidance.com and navigate to the Software Updates page for the software downloads.

You can also download the STX User Guide from the Terminals Technical Documentation section at www.OutbackGuidance.com or call Outback Customer Service at (800) 247-3808.

# Installation Instructions for eDrive XD/eDrive XC v1.7

- Download Chuckwalla_Update_1_7_1_1_37901.zip from the Outback Guidance Dealer Download Center and save it to your computer.

- Unzip the software by double clicking the Chuckwalla_Update_1_7_1_1_37901 file.

- Copy the Chuckwalla_Update_1_7_1_1_37901 file to the root of a cleanly formatted fat USB drive. (Note: DO NOT put this file in a folder. It must be on the root of the USB directory.)

- Rename the Chuckwalla_Update_1_7_1_1_37901 file to Chuckwalla_Update.

- Insert the usb thumb-drive into the un-powered eDriveXD / eDriveXC cable's USB connector.

- Power on the eDriveXD / eDriveXC.

- The USB recovery process will take approximately 10 minutes

- Wait for both LEDs to start flashing.

- Power Cycle the eDriveXD / eDriveXC connect to a terminal and verify the software update was successful.

  - Note – If no LED indicators are given wait 15 minutes and power cycle ECU and try to establish connection to a terminal and verify failure.

  - Note – If recovery update fail try different thumb drive.