| To: | eDriveXD / eDriveXC Users | Date: | July 21, 2021 |
| --- | --- | --- | --- |
| From: | Global Customer Care | Subject: | eDriveXD / eDriveXC 1.7.1.1.37901 Installation Instructions |

Product(s) Affected:

eDriveXD / eDriveXC

Scope:

This document provides software installation instructions for Outback eDriveXD / eDriveXC version 1.7.1.1.37901. This software allows the user to update the steering ECU’s software without being connected to an Outback terminal.

To take advantage of the improvements listed above, visit outbackguidance.zendesk.com/hc/en-us and navigate to the Software Updates page for the software downloads.

# Installation Instructions for eDrive XD/eDrive XC v1.7.1.1.37901

- Download Chuckwalla_Update_1_7_1_1_37901.zip from the Outback Guidance Dealer Download Center and save it to your computer.

- Unzip the software by double clicking the Chuckwalla_Update_1_7_1_1_37901 file.

- Copy the update_package_1_7_1_1_37901.updt file to the root of a cleanly formatted fat USB drive. (Note: DO NOT put this file in a folder. It must be on the root of the USB directory.)

- Insert the usb thumb-drive into the un-powered eDriveXD / eDriveXC cable's USB connector.

- Power on the eDriveXD / eDriveXC.

- The USB recovery process will take approximately 10 minutes

- Wait for both LEDs to start flashing.

- Power Cycle the eDriveXD / eDriveXC connect to a terminal and verify the software update was successful.

  - Note – If no LED indicators are given wait 15 minutes and power cycle ECU and try to establish connection to a terminal and verify failure.

  - Note – If recovery update fail try different thumb drive.